# Polypropylene-Fiber Composite Mechanical Property Prediction

(Developed by Israrul H. Hashmi, Rahul Karmakar and Tarak Patra, IIT Madras; for internal use at HPCL Green R&D Center)

## Overview

This project predicts the mechanical properties of polypropylene (PP) fiber-reinforced composites using three machine learning models: XGBoost (XGB), Gaussian Process Regression (GPR), and a Deep Neural Network (DNN). The models are trained on experimental data (dataset.xlsx) and estimate four key properties:

- Tensile Strength at Yield
- Elongation at Yield
- Ultimate Tensile Strength
- Elongation at Break

Users only need to provide sample compositions to receive all model predictions automatically. After setup, the process runs via a single command without any GUI or manual Python interaction.

## Prerequisites

Ensure that Python 3.x is installed. We recommend using Anaconda or Miniconda to manage the environment. Install the required libraries using Conda or pip:

- Install Python Using Conda:

- Install required Python packages:

- pip install numpy pandas xgboost scikit-learn tensorflow openpyxl

Comment: Above command line has to be run to install packages.

## Input Data

(This is also same as previous Approach.)

Prepare your input compositions in the datafile.xlsx file. This Excel file must contain exactly 7 columns with the following headers:

- PP_H200MA

- PP_H350FG

- PP_B120MA

- PP_C320MN

- GF

- MAPP

- CF

Each row in datafile.xlsx represents a different composite sample. All values should be fractions (e.g., 0.25) or proportions, and each row must sum to 1.0 (total composition equals 100%). Example:

| PP_H200MA | PP_H350FG | PP_B120MA | PP_C320MN | GF | MAPP | CF |
| --- | --- | --- | --- | --- | --- | --- |
| 0.50 | 0.30 | 0.10 | 0.00 | 0.10 | 0.00 | 0.00 |
| 0.25 | 0.25 | 0.25 | 0.25 | 0.00 | 0.00 | 0.00 |

Do not modify dataset.xlsx, which contains the training data for the models.

Until unless if we must add more data points gathered from experiments and literature, dataset.xlsx should not be modified.

## Running the Prediction Script

Execute the prediction script from the command line:

python predict_allmodel.py

This script will automatically:

- Load the training data from dataset.xlsx and train the three models (XGB, GPR, DNN).
- Read the sample compositions from datafile.xlsx.
- Generate predictions for each of the four properties for each model.
- Save the results to output.xlsx.

No interactive input is required beyond this command.

## Output

After the script finishes, check the file final_output.xlsx in the same directory. This file contains 12 columns of results (3 models × 4 properties). The columns are grouped by model as follows:

- XGB model predictions:

  - Tensile Strength at Yield

  - Elongation at Yield

  - Ultimate Tensile Strength

  - Elongation at Break

- GPR model predictions:

  - Tensile Strength at Yield

  - Elongation at Yield

  - Ultimate Tensile Strength

  - Elongation at Break

- DNN model predictions:

  - Tensile Strength at Yield

  - Elongation at Yield

  - Ultimate Tensile Strength

  - Elongation at Break

Each column in output.xlsx has a header indicating the model and property. Each row corresponds to the input sample from datafile.xlsx.

## Disclaimer

Confidential: This document and the associated tool are for internal use at HPCL Green R&D Center only. Do not share the software, documentation, or results with any unauthorized third parties.